RTL-base  ·  Timer Module

timer.h / timer.c  —  RTL8721Dx / KM4 (Cortex-M55)

# 1. Overview

The Timer module provides the foundational timing infrastructure for RTL-base. It fuses the Ameba GTimer (1 µs resolution, 64-bit via overflow tracking) with the ARM Cortex-M55 DWT CYCCNT (~5 ns resolution) to deliver a composite nanosecond-precision clock that is safe in both task and ISR contexts.

Key characteristics:

- Composite timestamp: GTimer provides the stable microsecond epoch; DWT provides sub-microsecond interpolation.

- Software PLL drift corrector: samples calibrate phase offset and measure DWT drift in ppm relative to the GTimer reference.

- Wrap watchdog task: detects DWT 32-bit counter wrap events and increments a software-extended 64-bit counter.

- ISR-safe: timer_get_time_ns_isr() is identical to the fine path — no locking, no OS calls.

- Fallback chain: DWT → SysTick → GTimer-only, selected at runtime based on hardware availability.

# 2. Architecture

## 2.1  Timestamp Composite

The fine timestamp is computed by the following pipeline:

gtimer_us_64()   →  stable 64-bit microsecond epoch

DWT_NOW()        →  cycle counter snapshot (taken before & after GTimer read)

phase offset     →  DWT phase calibrated at µs tick boundary

sub_ns = (midpoint_cycles - phase) * 1000 / cycles_per_us

result_ns = us_epoch * 1000 + sub_ns

The GTimer is read inside a double-read retry loop (up to 4 attempts) to guard against the read straddling a GTimer µs boundary. If no stable reading is obtained, the function falls back to the raw µs value.

## 2.2  Phase Calibration

timer_calibrate() performs TIMER_CAL_SAMPLES (default: 8) measurements. For each sample it waits for a GTimer µs edge, then captures the DWT counter. The phase is the DWT value modulo cycles_per_us — i.e., the cycle offset from the start of the current µs.

A circular-mean algorithm handles phase wrap-around correctly. The drift is computed from the span of the first and last calibration samples:

drift_ppm = (actual_cycles - ideal_cycles) * 1e6 / ideal_cycles

ℹ  Calibration should be called once after timer_init() and optionally re-run after significant temperature changes. The watchdog task sets s_calibrated = false on overrun detection.

## 2.3  DWT Wrap Watchdog

At 345 MHz, the 32-bit DWT counter wraps approximately every 12.4 seconds. The watchdog task (dwt_watchdog_task) wakes every TIMER_WRAP_WATCHDOG_MS (4000 ms) via vTaskDelayUntil and detects wrap by comparing the current DWT value to the previous sample. Each wrap increments s_dwt_hi, extending the counter to 64 bits.

An overrun is flagged if the OS sleep was more than 20% longer than requested — indicating scheduler starvation that may have caused a missed wrap — in which case calibration is invalidated.

## 2.4  Precision Spin-Wait

timer_spin_until_ns() busy-waits until an absolute nanosecond deadline. When the DWT is available and calibrated, it converts the delta to cycles and spins on the DWT counter using wrap-safe signed arithmetic:

while ((int32_t)(deadline_cyc - DWT_NOW()) > 0) __asm__("isb");

The ISB instruction prevents the CPU from speculating past the loop and ensures the load of CYCCNT is not reordered. Without DWT, the fallback spins on gtimer_us_64() at 1 µs granularity.

# 3. Initialisation Sequence

The required call sequence is:

timer_init();        // detect hardware, start GTimer, enable DWT

timer_calibrate();   // measure phase, compute drift, start watchdog

// optional:

timer_wrap_test();   // verify watchdog fires within 2× watchdog period

ℹ  timer_init() is idempotent — subsequent calls return immediately. injectorManager_create() checks timer_is_ready() and will refuse to create a manager if the timer has not been initialised.

# 4. API Reference

## 4.1  Initialisation

| Function | Signature | Description |
| --- | --- | --- |
| timer_init | void timer_init(void) | Hardware init: GTimer, DWT/SysTick detection. Idempotent. |
| timer_calibrate | void timer_calibrate(void) | Phase + drift calibration. Spawns watchdog task. |
| timer_wrap_test | bool timer_wrap_test(void) | Injects a wrap event; returns true if watchdog detected it. |

## 4.2  Timestamp Accessors

| Function | Signature | Description |
| --- | --- | --- |
| timer_get_time_ns | uint64_t (void) | Coarse timestamp: GTimer µs × 1000. Always available post-init. |
| timer_get_time_ns_fine | uint64_t (void) | Fine timestamp with DWT sub-µs interpolation. Task context. |
| timer_get_time_ns_isr | uint64_t (void) | ISR-safe alias of the fine path. No OS calls. |
| timer_get_time_us | uint64_t (void) | Raw GTimer 64-bit microsecond value. |
| timer_align_ns | uint64_t (uint64_t period_ns) | Returns next aligned nanosecond boundary. |

## 4.3  Busy-Wait Primitives

| Function | Signature | Description |
| --- | --- | --- |
| timer_spin_until_ns | void (uint64_t target_abs_ns) | Busy-waits until absolute nanosecond deadline. DWT or GTimer fallback. |
| timer_spin_for_ns | void (uint64_t duration_ns) | Busy-waits for relative duration. Bounded by TIMER_SPIN_MAX_NS. |

## 4.4  Unit Conversion

| Function | Signature | Description |
| --- | --- | --- |
| timer_ns_to_cycles | uint64_t (uint64_t ns) | Converts nanoseconds to DWT cycles. Integer, no FP. |
| timer_cycles_to_ns | uint64_t (uint64_t cycles) | Converts DWT cycles to nanoseconds. Integer, no FP. |
| timer_get_cycles32 | uint32_t (void) | Raw 32-bit DWT CYCCNT read. |
| timer_get_cycles64 | uint64_t (void) | 64-bit DWT via double-read hi/lo retry. |

## 4.5  Diagnostics

| Function | Signature | Description |
| --- | --- | --- |
| timer_is_ready | bool (void) | Returns true after timer_init(). |
| timer_is_calibrated | bool (void) | Returns true after successful timer_calibrate(). |
| timer_get_resolution_ns | uint32_t (void) | Effective resolution: ~5 ns (DWT), ~1000 ns (GTimer). |
| timer_get_resolution_source | const char* (void) | "dwt", "systick", or "gtimer". |
| timer_get_cpu_freq_hz | uint32_t (void) | CPU frequency used for cycle conversion. |
| timer_get_dwt_phase | uint32_t (void) | Calibrated DWT phase offset (cycles into µs). |
| timer_get_drift_ppm | int32_t (void) | DWT drift relative to GTimer in ppm. |
| timer_get_wrap_count | uint32_t (void) | Number of DWT 32-bit wraps detected. |
| timer_get_overrun_count | uint32_t (void) | Number of watchdog overrun events. |

# 5. Configuration Macros

| Macro | Default | Description |
| --- | --- | --- |
| TIMER_GTIMER_IDX | TIMER0 | GTimer hardware index |
| TIMER_CAL_SAMPLES | 8 | Number of calibration edge samples. Range [4, 64]. |
| TIMER_SPIN_MAX_NS | 10 000 000 | Maximum spin duration (ns). Clamped to < 6 s. |
| TIMER_SPIN_MIN_NS | 100 | Minimum spin duration. Shorter waits are skipped. |
| TIMER_WRAP_WATCHDOG_MS | 4000 | Watchdog wake period. Must be < 5 000 ms at 200 MHz. |

# 6. Thread Safety & ISR Contracts

All timer read functions are reentrant — they contain no global writes and no OS calls. The only global state mutations are in:

- timer_init() — protected by taskENTER_CRITICAL / taskEXIT_CRITICAL.

- timer_calibrate() — serialised by the s_calibrating flag; may yield (taskYIELD) between samples.

- dwt_watchdog_task — increments s_dwt_hi and s_gtimer_hi inside a critical section.

ℹ  timer_spin_until_ns() disables no interrupts and holds no locks. It is safe to call from a task that holds a mutex, but note it will block the CPU (not the task) for the duration — only use for sub-10 ms waits.

# 7. Usage Example

// Initialise once at startup

timer_init();

timer_calibrate();

// Obtain a fine timestamp

uint64_t t0 = timer_get_time_ns_fine();

do_work();

uint64_t elapsed_ns = timer_get_time_ns_fine() - t0;

// Spin until an absolute deadline

uint64_t deadline = timer_get_time_ns_fine() + 500000ULL; // +500 us

timer_spin_until_ns(deadline);

// Check timer health

printf("source=%s  drift=%d ppm  wraps=%u",

timer_get_resolution_source(),

timer_get_drift_ppm(),

timer_get_wrap_count());