RTL-base  ·  Monitor Module

monitor.h / monitor.c  —  RTL8721Dx / KM4 (Cortex-M55)

# 1. Overview

The Monitor module implements a non-blocking Wi-Fi promiscuous capture engine that filters, buffers, and streams 802.11 frames to a host PC in pcapng, pcap, or raw .cap format. The capture pipeline is deliberately split into three FreeRTOS tasks with lock-free data paths to minimise latency and avoid priority inversion on the ISR path.

Architecture summary:

- promisc_callback (ISR context): extracts frame from SDK rx_pkt_info, allocates a slot from the lock-free packet pool, timestamps it with timer_get_time_ns_isr(), and pushes a descriptor into the SPSC ring buffer.

- monitor_task (task context): drains the ring, applies all filters (RSSI threshold, frame type, MAC, BPF), updates per-channel statistics, invokes the optional capture callback, and serialises pcapng EPBs into the block pool.

- writer_task (task context): receives block pointers from a FreeRTOS queue and calls the user-supplied output sink (default: blocking UART byte-by-byte).

- hopper_task (task context): manages channel switching with adaptive dwell times derived from per-channel traffic EMA.

# 2. Data Path

## 2.1  Packet Pool (Lock-Free)

PACKET_POOL_SIZE (default 64) fixed-size slots of PACKET_BUFFER_SIZE (2346) bytes each are managed by an SPSC free-index ring. Allocation and free are O(1) atomic load/store operations. The pool is safe for ISR allocation and task-context free — the ISR only writes the write pointer, the task only reads and restores to the free ring.

ℹ  The pool uses unsigned wrap arithmetic for head/tail. Overflow of the 32-bit indices after ~4 billion operations is harmless because the difference (used count) remains correct.

## 2.2  SPSC Ring Buffer

The ring holds RING_SIZE (1024) rb_entry_t descriptors (32 bytes each). Size is enforced to be a power of two by a _Static_assert. The ISR writes s_write_idx after filling a slot; monitor_task reads s_read_idx. No locks are used — the single-producer/single-consumer constraint holds because the promisc callback is called serialised by the Wi-Fi driver.

## 2.3  Block Pool and Writer Queue

The writer path uses a separate block pool (BLOCK_POOL_SIZE = 32 slots, BLOCK_MAX_SIZE = 2448 bytes) to avoid per-packet heap allocation on the hot writer path. Each slot has an atomic in_use flag; block_alloc() scans for the first free slot using CAS. Completed blocks are pointer-queued to writer_task via a FreeRTOS queue of depth WRITER_QUEUE_LEN (128).

## 2.4  Output Format Selection

The output format is selected at compile time via -DMONITOR_FORMAT=:

| Macro Value | Format | Notes |
| --- | --- | --- |
| 0 (CAP) | Raw 802.11 | pcap global header + raw frames. No radiotap. |
| 1 (PCAP) | libpcap | pcap global header + 24-byte radiotap + frames. LINKTYPE 127. |
| 2 (PCAPNG) | pcapng | SHB + IDB (ns resolution, FCS opt) + EPBs. Default. Wireshark-ready. |

# 3. Filtering Pipeline

Filters are evaluated in monitor_task in the following order. A packet is dropped if any filter rejects it:

| Stage | API | Logic |
| --- | --- | --- |
| RSSI gate | monitor_set_rssi_threshold() | Drop if rssi < min_rssi. Default: -128 dBm (pass all). |
| Frame type | monitor_set_filter() | FILTER_ALL / DATA / MANAGEMENT / CONTROL / BEACON / PROBE_REQ / PROBE_RSP. |
| MAC filter | monitor_set_mac_filter_mode() | ALLOWLIST: pass only listed DA/SA/BSSID. DENYLIST: drop listed addresses. |
| BPF rules | monitor_set_bpf_rules() | Up to MAX_BPF_RULES (8) byte-mask-value matchers evaluated conjunctively. |

MAC address matching checks Destination Address (offset 4), Source Address (offset 10 if frame type has A2), and BSSID (offset 16 if frame type has A3). Control frames with no A2 field are matched only on DA.

ℹ  All filter state is written from task context only. Reads in monitor_task are also in task context. No synchronisation is required beyond the volatile qualifier on scalar fields.

# 4. Channel Hopping

## 4.1  Adaptive Dwell

The hopper_task iterates the 38-channel list (13 × 2.4 GHz + 25 × 5 GHz). After each channel switch it waits for a computed dwell time:

traffic_ema = 7/8 * traffic_ema + 1/8 * 256  (on every captured frame)

dwell_ms = HOP_DWELL_MIN_MS + (HOP_DWELL_MAX_MS - HOP_DWELL_MIN_MS) * ema / threshold

This means channels with more traffic are dwelt on longer (up to 300 ms), while idle channels are visited for the minimum dwell (50 ms). After computing the dwell, the EMA is decayed (×7/8) to allow the channel to shorten again if traffic drops.

## 4.2  Fixed Channel Mode

Calling monitor_set_fixed_channel(ch) locks the hopper to a single channel. monitor_set_hopping() restores sweep mode. Channel switches still pass through hopper_task — the injector and monitor share the same radio via the Wi-Fi driver.

# 5. Per-Channel Statistics

channel_stats_t is accumulated for each of the 38 channels in s_ch_stats[]. Fields:

- frames_total / frames_mgmt / frames_ctrl / frames_data / frames_other: cumulative frame counts.

- rssi_min / rssi_max / rssi_sum: RSSI range and sum.

- rssi_m2: Welford M2 accumulator for online variance computation (variance = rssi_m2 / (n-1)).

- rssi_histogram[8]: RSSI distribution in 8 buckets: <-90, -90 to -80, ..., >-30 dBm.

- traffic_ema: exponentially weighted frame rate used by the hopper dwell algorithm.

# 6. API Reference

## 6.1  Lifecycle

| Function | Returns | Description |
| --- | --- | --- |
| monitor_start | void | Initialises pool, creates 3 tasks, writes session header, enables promisc mode. |
| monitor_stop | void | Sets running=0, disables promisc, waits for all tasks, drains pool. |

## 6.2  Channel Control

| Function | Returns | Description |
| --- | --- | --- |
| monitor_set_fixed_channel | void | Fix hopper to channel ch (1–13, 36–165). |
| monitor_set_hopping | void | Resume 38-channel adaptive sweep. |
| monitor_get_channel | uint8_t | Returns the currently tuned channel. |

## 6.3  Output Sink

| Function | Returns | Description |
| --- | --- | --- |
| monitor_set_output_sink | void | Installs custom sink callback. NULL restores UART default. |

## 6.4  Filters

| Function | Returns | Description |
| --- | --- | --- |
| monitor_set_filter | void | Set frame-type filter. Default: FILTER_ALL. |
| monitor_set_rssi_threshold | void | Drop frames below min_rssi dBm. Default: -128. |
| monitor_set_mac_filter_mode | void | MAC_FILTER_NONE / ALLOWLIST / DENYLIST. |
| monitor_add_mac_filter | int | Add 6-byte MAC to filter list (max MAX_MAC_ENTRIES). |
| monitor_remove_mac_filter | int | Remove MAC from list. Returns -1 if not found. |
| monitor_clear_mac_filter | void | Clears all MAC entries. |
| monitor_set_bpf_rules | void | Install up to MAX_BPF_RULES byte-mask-value rules. |
| monitor_clear_bpf_rules | void | Clear all BPF rules. |

## 6.5  Statistics

| Function | Returns | Description |
| --- | --- | --- |
| monitor_get_channel_stats | int | Copies channel_stats_t for the given channel. |
| monitor_reset_channel_stats | void | Zeroes all 38 channel stat structures. |
| monitor_get_stats | void | Returns global captured/dropped counters via out-pointers. |
| monitor_reset_stats | void | Resets all global counters to zero. |
| monitor_set_capture_callback | void | Per-frame callback: (frame, len, channel, rssi, ts_ns). Task context. |

# 7. Configuration Macros

| Macro | Default | Description |
| --- | --- | --- |
| MONITOR_FORMAT | 2 (PCAPNG) | Output format: 0=CAP, 1=PCAP, 2=PCAPNG. |
| WLAN_IDX | STA_WLAN_INDEX | Wi-Fi interface index. |
| PACKET_POOL_SIZE | 64 | Number of packet pool slots. Max 255 (uint8_t index). |
| WRITER_QUEUE_LEN | 128 | Depth of the block pointer queue. |
| MAX_MAC_ENTRIES | 16 | Maximum MAC filter list entries. |
| MAX_BPF_RULES | 8 | Maximum BPF rules. |
| IDB_FCS_LEN_BYTES | 0 | FCS length reported in pcapng IDB option (0 or 4). |
| HOP_DWELL_MIN_MS | 50 | Minimum channel dwell time (ms). |
| HOP_DWELL_MAX_MS | 300 | Maximum channel dwell time (ms). |
| HOP_TRAFFIC_HIGH_THR | 20 | Traffic EMA threshold for maximum dwell. |

# 8. Usage Example

// Install a custom USB-CDC sink

void usb_sink(const void *data, size_t len) {

usb_cdc_write((const uint8_t*)data, len);

}

monitor_set_output_sink(usb_sink);

// Capture only beacons from a specific BSSID

monitor_set_filter(FILTER_BEACON);

monitor_set_mac_filter_mode(MAC_FILTER_ALLOWLIST);

const uint8_t bssid[] = {0xAA,0xBB,0xCC,0x11,0x22,0x33};

monitor_add_mac_filter(bssid);

monitor_set_rssi_threshold(-75);  // only strong signals

// Start capture on channel 6

monitor_set_fixed_channel(6);

monitor_start();

// Check drop counters after 10 s

vTaskDelay(pdMS_TO_TICKS(10000));

uint32_t cap, dr, dp, pk, db;

monitor_get_stats(&cap, &dr, &dp, &pk, &db);

printf("captured=%u dropped_ring=%u dropped_pool=%u peak_pool=%u\n",

cap, dr, dp, pk);

monitor_stop();